PRETEST

1. The speed of a DC shunt motor is directly proportional to:

Answer: c) Back emf and inversely proportional to flux

2. Which of the following equations represents the speed of a DC motor?

Answer: a) N = V−IaRa/Kφn

3. In the field control method of speed control, speed is varied by:

Answer: c) Changing field current

4. When the field resistance of a DC shunt motor is increased, the speed:

Answer: a) Increases

5. The field control method is suitable for:

Answer: a) Speeds above rated speed

6. The torque of a DC motor is proportional to:

Answer: c) Product of armature current and flux

7. The flux in a DC motor is directly proportional to:

Answer: b) Field current

8. Which component controls the field current in a DC shunt motor?

Answer: b) Field rheostat

9. Which of the following is a disadvantage of the field control method?

Answer: d) All of the above

10. The speed of a DC motor increases if the field current is:

Answer: b) Decreased

11. If field resistance is increased too much, the motor may:

Answer: c) Run dangerously fast

12. The field control method is also known as:

Answer: c) Flux control method

13. In the field control method, power loss is:

Answer: b) Low

14. What will happen if the field winding of a DC motor opens during operation?

Answer: b) Motor speed will increase uncontrollably

15. Why is the field control method preferred for light loads?

Answer: d) Because the torque requirement is low

16. In field control of a DC shunt motor, which parameter is intentionally weakened to increase speed?

 Answer: b) Field flux

17. Field control method is most effective in which type of DC motor?

 Answer: b) Shunt motor

18. Which of the following remains approximately constant during field control speed regulation?

 Answer: b) Supply voltage

19. In field control, weakening the field mainly affects which motor characteristic?

 Answer: c) Torque capability

20. Field control is generally not suitable for heavy loads because:

 Answer: c) Torque reduces with flux weakening

POSTTEST

1. What happens to the speed of a DC shunt motor when field resistance is increased?

Answer: b) Speed increases

2. The relation between speed (N) and flux (Φ) for a DC motor is:

Answer: b) N∝1/Φ

3. During the experiment, what variable is controlled to change the motor speed?

Answer: b) Field current

4. When the field current decreases, the flux:

Answer: b) Decreases

5. The field rheostat in the experiment is used to:

Answer: b) Vary the field current

6. In the speed vs field current curve obtained experimentally, the curve is:

Answer: c) Hyperbolic

7. Why does the motor speed increase when the field resistance is increased?

Answer: c) Flux decreases

8. In the field control method, the torque of the DC motor:

Answer: b) Decreases with a decrease in field current

9. What happens if the field circuit opens while the motor is running?

Answer: c) Motor speed rises dangerously

10. The experimental readings typically include measurements of:

Answer: a) Field current, armature current, speed

11. The constant k in the speed equation N = V−IaRa/kΦN depends on:

Answer: a) Motor construction

12. The maximum safe operating speed of the DC motor is called:

Answer: a) Rated speed

13. If the field resistance is decreased, the motor speed will:

Answer: b) Decrease

14. What is the typical range of speed control achieved using the field resistance method?

Answer: b) Above rated speed

15. What safety precaution must be ensured during the experiment?

Answer: b) Do not open the field circuit while the motor is running

16. From the experimental results, if the field current is reduced by 50%, the motor speed will:

Answer: c) Increase significantly depending on flux

17. Which conclusion can be drawn from the speed vs field current graph obtained in the experiment?

Answer: c) Speed is inversely proportional to field current

18. Why is field control not suitable for precise low-speed control?

Answer: b) Because flux cannot be increased beyond rated value

19. In the experiment, weakening the field beyond a limit may lead to:

Answer: c) Mechanical stress and unsafe operation

20. Based on the experiment, which statement is correct for practical applications?

Answer: b) Field control is suitable for high-speed, light-load operation